TU/CDOE

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (AUTUMN) 2020

DRE 201: ENERGY MANAGEMENT AND AUDITING

Time: 3 hours                                               Full Marks: 70

The figures in the right-hand margin indicate marks for the individual questions

**************************************************************

- Choose the correct option:                                                      2x10= 20

- In a purely resistive circuit, the reactive kVAr will be-

i. Less than one		ii. more than one

iii. Zero			iv. One

- TOD is electrical utility refers to-

i. Time of day		ii.  Total operating demand

iii. Total outages per day	iv. Time of demand

- Which of the following is not a part of Energy Monitoring?

i. Recording			ii. Analysing

iii. Reporting			iv. Financing

- Power factor is the ratio of -

i. kVA/kW			ii.  kW/kVA

iii. kVAr/kVA 		iv. kVA/kVAr

- Energy supplied by combustion of fuel is equal to-

i. fuel mass x density                ii.  fuel mass x specific heat

iii. fuel mass x calorific value  iv. fuel mass x heat capacity

- A firm substitutes (replaces) expensive oil and starts

firing wood in a boiler. The following scenario is most

likely-

i. Higher energy consumption  ii. higher fuel cost

iii. better boiler efficiency	   iv. less smoke in the stack

- The ratio of energy consumption to corresponding

production is called-

i. specific energy consumption  ii. Energy intensity

iii. production energy ratio 	    iv. specific production ratio

P.T.O.

- Excess air can be derived by measuring percentage of_____ in flue gas-

i. CH4         	 		     ii. O2

iii. CO            			     iv.  N2

- Energy consumed for the period is described by the

following equation (C= fixed and M is variable energy

consumption)-

i. C+M ×Production		     ii. C × M + Production

iii. (C+M) × Production 		     iv. C × M × Production

- CPM of a project management refers to-

i. Critical periodic management     ii.  critical path method

iii. cost project management            iv. critical project

monitoring

2. Answer the following:                                                 3+3+4+4+6=20

- Explain the difference between energy conservation and energy efficiency with suitable examples.

- Write down the reporting format of an Energy Conservation Recommendation in Energy Auditing Report.

- An oil fired boiler with a rated capacity of 12 tonne/h steam generation is switched over to rice husk firing. The boiler is de-rated to 7 tonne/h. List down four major reasons for the de-rating.

- Explain briefly: (i) guaranteed saving performance contract and (ii) shared savings performance contract.

- In a textile plant, the average monthly energy consumption is 7 lakhs unit of electricity from grid, 40 kL of furnace oil (specific gravity: 0.92) for thermic fluid heater, 60 tonne of coal for steam boiler and 10 kL of HSD (specific gravity: 0.885) for material handling equipment. Calculate the energy consumption in terms of Metric Tonne of Oil Equivalent (MTOE) for the plant. Comment whether this textile plant qualifies as a notified designated consumer under the Energy Conservation Act?

(Given data: 1 kWh=860 kcal, GCV of coal: 3450 kcal/kg,

GCV of furnace oil: 10, 600 kcal/kg, GCV of HSD: 10,500

kcal/kg and 1 kg oil equivalent: 10000 kcal.)

3. Answer the following:                                                                3+6+6=15

- Explain TOD tariff and how it is beneficial for the power system and consumers?

- An energy meter connected to a 3 phase, 18.75 kW pump shows 108 units’ consumption for six hours of operation. The load on the motor was steady. The consumer doubted the energy meter reading and electrical parameter such as current, voltage and power factor were measured. The measured values were 	430 V line volts, 25 A line current and 0.80 Power Factor. Find out if the energy meter reading is correct.

- In an industry workshop 80 numbers of fused 60 W incandescent light bulbs (ILB) are replaced by same numbers of 15 W CFL instead of new ILB. Calculate the following for 4000 hours of operation per year.

  - Annual “kWh saved”

  - “annual kVAh saved” if the power factor of the CFL is 0.6,

  - annual reduction in electricity costs if Rs. 5 per kWh is the energy charge and Rs. 250 per kVA per month is the demand charge.

4. Answer the following:                                                              4+5+6=15

- How do you calculate the efficiency of a boiler by indirect method?

- Discuss four major energy conservation opportunities in a boiler system.

- During April 2017, a plant has recorded a maximum demand of 600 kVA and average power factor (PF) is observed to be 0.82 lag. The minimum PF to be maintained is 0.92 lag as per the utility regulations and every one % dip (or decimal) in PF attracts a penalty of Rs 10,000 in each month. Calculate the improvement in PF for May 2017 by installing 100 kVAr capacitors and also the differences in penalty paid in April and to be pay if any, during May 2017.

***