TU/CDOE

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (AUTUMN) 2020

DRE 202 : ENERGY EFFICIENCY IN THERMAL UTILITIES

Time: 3 hours                                                            Full Marks: 70

The figures in the right-hand margin indicate marks for the individual questions

*****************************************************************

1. Fill in the blanks.			                                          1×10=10

- The lowest temperature required to make the combustion self-sustained without any external aid is known as the __________ temperature.

- __________ point indicates the maximum temperature at which liquid fuel can be stored without any fire hazard.

- In __________ circulation boilers, the flow is caused by means of density difference of hot and cold water.

- Discharging of water from boiler for removal of the suspended solids and bottom sludge from the boilers is known as __________.

- __________ steam is produced when condensate at a high pressure is released to a lower pressure and can be used for low pressure heating.

- The __________ diagram is used to visualise the magnitude of flow, between different nodes in a network, through the width of the arrows.

- In a __________ cycle, the primary fuel is first used to produces high temperature thermal energy and the heat rejected from the process is used to generate power through a recovery boiler and a turbine generator.

- __________ systems are designed to recover the unused heat to do some useful work.

- __________ materials are semiconductor solids that allow direct generation of electricity when subjected to a temperature differential.

- The effectiveness of a heat exchanger depends on the geometry of the heat exchanger as well as the __________ arrangement.

P.T.O.

TU/CDOE

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (AUTUMN) 2020

DRE 202 : ENERGY EFFICIENCY IN THERMAL UTILITIES

Time: 3 hours                                                            Full Marks: 70

The figures in the right-hand margin indicate marks for the individual questions

*****************************************************************

1. Fill in the blanks.			                                          1×10=10

- The lowest temperature required to make the combustion self-sustained without any external aid is known as the __________ temperature.

- __________ point indicates the maximum temperature at which liquid fuel can be stored without any fire hazard.

- In __________ circulation boilers, the flow is caused by means of density difference of hot and cold water.

- Discharging of water from boiler for removal of the suspended solids and bottom sludge from the boilers is known as __________.

- __________ steam is produced when condensate at a high pressure is released to a lower pressure and can be used for low pressure heating.

- The __________ diagram is used to visualise the magnitude of flow, between different nodes in a network, through the width of the arrows.

- In a __________ cycle, the primary fuel is first used to produces high temperature thermal energy and the heat rejected from the process is used to generate power through a recovery boiler and a turbine generator.

- __________ systems are designed to recover the unused heat to do some useful work.

- __________ materials are semiconductor solids that allow direct generation of electricity when subjected to a temperature differential.

- The effectiveness of a heat exchanger depends on the geometry of the heat exchanger as well as the __________ arrangement.

P.T.O.

2. Best quality coal is required for an industry. How will you select the coal from a variety of types available in the markets? Also discuss the procedure of evaluation of performance of a boiler which uses coal as fuel. 	    				    	             5+5 = 10

3.  How will you describe the functioning of a steam power plant to a technical visiting team? Also provide sketches showing the different processes taking place in the steam power plant.                       5+5 = 10

4.  A considerable amount of heat is wasted in a plant having thermal utilities.  Explain the benefits of waste heat recovery describing some practical methods of waste heat recovery.                                      6+4=10

5. Mention the important factors to be considered for selecting size of pipe i.e., diameter of a pipe to carry steam. Explain the importance of steam trap and mention the different types of steam trap.        6+4=10

6. With the help of neat sketch explain the uses of boiler accessories. Also mention the importance of maintenance required in a boiler.

5+5= 10

7. The flow rates of hot and cold-water streams running through a parallel-flow heat exchanger are 0.4 kg/s and 1.0 kg/s, respectively. The inlet temperatures on the hot and cold sides are 75°C and 25°C, respectively. The exit temperature of hot water is 50°C. Calculate the log mean temperature difference for the above case. Do you believe that change of flow rates could change the LMTD? Justify your answer.                                                                                                                    5

8. A gas consists of 60% propane (C3H8) and 40% butane (C4H10) by volume. Determine the stoichiometric air-to-fuel ratio. Also state what will happen if excess air is used. 	                                                         5

***

2. Best quality coal is required for an industry. How will you select the coal from a variety of types available in the markets? Also discuss the procedure of evaluation of performance of a boiler which uses coal as fuel. 	    				    	             5+5 = 10

3.  How will you describe the functioning of a steam power plant to a technical visiting team? Also provide sketches showing the different processes taking place in the steam power plant.                       5+5 = 10

4.  A considerable amount of heat is wasted in a plant having thermal utilities.  Explain the benefits of waste heat recovery describing some practical methods of waste heat recovery.                                      6+4=10

5. Mention the important factors to be considered for selecting size of pipe i.e., diameter of a pipe to carry steam. Explain the importance of steam trap and mention the different types of steam trap.        6+4=10

6. With the help of neat sketch explain the uses of boiler accessories. Also mention the importance of maintenance required in a boiler.

5+5= 10

7. The flow rates of hot and cold-water streams running through a parallel-flow heat exchanger are 0.4 kg/s and 1.0 kg/s, respectively. The inlet temperatures on the hot and cold sides are 75°C and 25°C, respectively. The exit temperature of hot water is 50°C. Calculate the log mean temperature difference for the above case. Do you believe that change of flow rates could change the LMTD? Justify your answer.                                                                                                                    5

8. A gas consists of 60% propane (C3H8) and 40% butane (C4H10) by volume. Determine the stoichiometric air-to-fuel ratio. Also state what will happen if excess air is used. 	                                                         5

***